SOP09-36 Wireless Network Procedures

CUI

Indian Health Service

Division of Information Security

Standard Operating Procedure for Wireless

Network Security Standards

DIS-SOP-09-36

Version 2.2

February 2013

CONTROLLED UNCLASSIFIED INFORMATION (CUI)

Controlled with Standard Dissemination

This information is subject to safeguarding measures that

reduce the risks of unauthorized or inadvertent disclosure.

Dissemination is permitted to the extent that it would further

the execution of a lawful or official purpose.

DIS-SOP-09-36 Wireless Network Security

Standards

Record of Changes

Version Date Name Description Section

1.0 05/31/04 DIS Final All sections

1.1 12/31/08 DIS Added additional

requirement for scanning

unauthorized WAPs

Sec 5

1.2 09/31/09 DIS Overall update All sections

2.0 11/01/11 P&P Team Applied SOP template;

added additional checklist

item; changed WLAN

scanning frequency; added

two wireless scanning tools;

changed ITAC Form to ITAC

System; added procedure for

disabling wireless card in

Windows 7

All

sections

2.1 12/19/12 P&P Team Added Contact section;

changed scanning frequency

from 18 months to annually;

added least privilege

requirement; updated

scanning tools in Appendix A

All

sections

2.2 8/21/14 P&P Team Removed procedure for XP;

applied new introduction

matter; updated wireless

scanning section (removed

reference to PTT service and

VisiWave); minor language

corrections throughout

Sections

1; 2.1

2.3 3/1/16 PSA Team Updated review schedule.

Updated color scheme.

Added document name to

header.

All

Indian Health Service  Version 2.2

Division of Information Security ii February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

Approval

This Standard Operating Procedure (SOP) has been approved for distribution and implementation. These

new procedures are effective immediately and will be enforced. Representatives of management will be

authorized to conduct periodic quality checks and audits to assure compliance with these procedures.

Requests for corrections or changes to any procedures should be sent to the IHS Chief Information

Officer (CIO).Exceptions or exemptions to any of these procedures must be submitted in writing to the

IHS CIO for approval or disapproval.

Approved by:

/  Howard Hays  / 2/1/13

Howard Hays, MD, MSPH

CAPT, U.S. Public Health Service

Chief Information Officer, IHS (Acting)

Date

Date of last review:

January 23, 2015 Reviewed by:

/  Steven Miller  /

________________________________

Steven Miller

Commander, U.S. Public Health Service

Supervisory IT Specialist, IHS

Indian Health Service  Version 2.2

Division of Information Security iii February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

Contents

1. Introduction ......................................................................................................................... 1

1.1 Purpose and Scope ......................................................................................... 1

1.2 Contact Information ......................................................................................... 3

2. Standards for Wireless Network Security ......................................................................... 4

2.1 Requirements .................................................................................................. 4

2.2 Recommendations .......................................................................................... 6

3. Guidelines for Implementing Wireless Networks at IHS .................................................. 8

4. Inappropriate Use ................................................................................................................ 9

5. Checklist ............................................................................................................................ 10

6. Glossary ............................................................................................................................. 11

7. Appendix: Wireless Scanning Tools ................................................................................ 12

Indian Health Service  Version 2.2

Division of Information Security iv February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

1. Introduction

The Indian Health Service (IHS) is responsible for implementing and

administering an information security program to protect the agency’s

information resources. The procedures in this document are designed to protect

IHS technology resources and data by mitigating information security threats,

improving incident response, and reducing interference among systems.

Wireless network devices offer a simple, convenient, and inexpensive way to

extend network accessibility by reducing the requirements of physical

infrastructure. Wireless networking eliminates wired connections on portable

devices and makes it possible for laptop and handheld users to travel beyond

traditional network boundaries (for example, between buildings) without losing

network connectivity.

However, in addition to the risks associated with any wired network, wireless

technology introduces several unique vulnerabilities. Because wireless signals are

radio transmissions, the proliferation of wireless appliances at IHS can lead to

increased radio frequency interference (RFI) among medical devices.

Furthermore, the signals can be intercepted by other radio receiving devices,

including those outside the intended service area. If data transmissions are

inadequately encrypted, the intercepted data can be read and understood in a

matter of seconds. That means any data transmission sent through the wireless

network is at risk, including patient data, user names and passwords, financial

data, and other sensitive information. Advances in wireless signaling technology

may increase transmission distances, further exacerbating the problem of

unauthorized reception.

Because wireless transmissions can circumvent traditional perimeter firewalls,

existing protections established to prevent unauthorized access are ineffective.

Websites that identify open access points (“hot spots”) that malicious users can

use to access a network are becoming more common. If the IHS wireless network

is improperly implemented, it can enable an unauthenticated user to get an IHS

Internet Protocol (IP) address with all the benefits of an authenticated network

user. Using one of these trusted IP addresses, a malicious user could attack IHS or

any outside network that is connected to IHS.

This SOP establishes the procedure for the secure deployment and use of wireless

network technology.

1.1 Purpose and Scope

This IHS Standard Operating Procedure (SOP) establishes wireless network

security standards and procedures for IHS, compliant with all applicable federal

laws and regulations. This procedure is, in large part, based on the National

Institute of Standards and Technology (NIST) Special Publication (SP) 800-153,

Guidelines for Securing Wireless Local Area Networks (WLANs). This SOP

Indian Health Service  Version 2.2

Division of Information Security 1 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

applies to all organizations or persons that collect, process, or maintain federally

owned information or information systems, either directly or on behalf of IHS.

• This procedure supersedes SOP 06-36, Wireless Network Security Standards,

but does not supersede any other applicable law or higher-level agency

directive or policy guidance.

• The content of and compliance with this procedure shall be incorporated into

applicable contract language, memoranda of agreement, or other agreements.

• This SOP pertains to all individuals using IHS systems, whether federal or

non-federal.

• Areas must use this SOP or create a more restrictive SOP, but cannot use one

that is less restrictive or less comprehensive, or does not comply with this

document, departmental guidelines, IHS policies, or federal regulation.

• This procedure will be reviewed every three years to ensure that it is current

and will expire three years from the date of its approval.

This SOP applies to all administrative management offices such as IHS

Headquarters, Area Offices and service units, and all IHS-funded or directly

supported health facilities and locations.

The term facilities includes all Indian Health urban locations where healthcare is

provided or managed and all IHS or Tribally owned or operated hospitals, clinics,

or any other locations where healthcare services are provided, managed, or

otherwise supported.

• Facilities managed by Tribes or Tribal organizations whose network services

IHS provides must follow this SOP in accordance with their contracts or

compacts. IHS recommends all facilities operated by Tribe follow this SOP

regardless of a given facility’s network service provider.

• Contractors’ and partners’ facilities must follow this SOP in accordance with

their interconnection security agreements with IHS.

For the purpose of this document, the term IHS includes any of the facilities

described above.

Indian Health Service  Version 2.2

Division of Information Security 2 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

1.2 Contact Information

If you have any questions or comments regarding this document, please contact

the Office of Information Technology (OIT) Help Desk:

Phone: 888-830-7280

Web: http://www.ihs.gov/helpdesk

Email: support@ihs.gov

Indian Health Service  Version 2.2

Division of Information Security 3 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

2. Sta ndards for Wireless Network Security

All IHS federal employees, contractors, and third-party organizations must adhere

to the following requirements and recommendations when they use wireless local

area networks (WLANs) on any IHS IT system.

2.1 Requirements

These standards outline appropriate use of the IEEE 802.11 WLANs for IHS. The

requirements are based on the IEEE 802.11 family of standards and do not

incorporate any proprietary vendor security controls.

• WLAN products must comply with IEEE 802.11-2007.

Standard Configuration

• Wireless endpoint communications must be disengaged while they are

physically connected to the IHS enterprise wired network. Some laptop

computers have a wireless switch on the side or the front that can be used to

disable and enable wireless functionality. The wireless switches usually have

an image of a wireless antenna on them. The images vary by manufacturer,

but they are usually similar to the examples shown in Figure 1.

Figure 1: Examples of Wireless Antenna Images

If the computer doesn’t have a wireless switch, follow the steps in the procedures

below to disable the wireless network connection in Windows 7.

NOTE: Windows XP is no longer supported by Microsoft and should not be

used at IHS. Windows 8 has not yet been formally authorized by IHS.

Windows 7

1. On the Start menu, click Control Panel.

2. Click Network and Internet.

3. Click Network and Sharing Center.

4. Click Change Network Adapter Settings.

Indian Health Service  Version 2.2

Division of Information Security 4 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

5. Right-click the wireless network connection that you want to disable, and then

click Disable, as shown in Figure 2.

Figure 2: Windows 7 Wireless Network Connection Menu

• WLANs configured for public or open access are untrusted networks and must be

segmented from the internal IHS network. A demilitarized zone (DMZ) must be

placed between the WLAN and its wired distribution system, and a compliant

Virtual Private Network (VPN) must be in place if a facility supports public

users/open access.

• IHS password requirements and the warning banner procedure must be applied to

all WLAN components. IHS password requirements are located in section 9 of the

IHS General User Security Handbook. The IHS warning banner procedure is

located in section 5.2 of the IHS Technical and Managerial Security Handbook.

• Designated personnel, such as the local ISSO, must scan WLANs annually or

when major changes are made in order to identify and remove any unauthorized

wireless access points. The WLAN’s entire coverage area must be scanned.

• Several free scanning tools are available for WLAN scanning. Section 7,

Appendix: Wireless Scanning Tools contains information about some of the many

available wireless scanning tools. WLAN coverage areas that use a wireless

intrusion detection system do not have to be scanned.

• Data in transit from authorized users on a WLAN to authorized users on a wired

LAN must be filtered through an appropriate filtering device (for example, a

firewall).

• WLAN traffic destined for the wired local area network must implement the

principle of least privilege requirements (for example, specific IP addresses,

protocols, ports, and hosts) to  reduce the risk of unauthorized access.

• Mai ntain an inventory of a ll access points (APs) and assign static IP addresses to

the APs. APs should be contained within enclosures or in physically secure

locations to reduce the risk of unauthorized physical tampering or unauthorized

physical access.

Indian Health Service  Version 2.2

Division of Information Security 5 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

• If Simple Network Management Protocol (SNMP) is turned on for management

purposes, change SNMP community strings from their manufacturer default to

unique and difficult-to-guess strings.

• WLANs that are not configured for open/public access must implement IEEE.

• Implement 802.1X access control to reduce the risk of access from unauthorized

WLAN devices, and provide authentication using a Federal Information

Processing Standards (FIPS)-140-validated and Wi-Fi Protected Access 2

(WPA2)-certified Extensible Authentication Protocol (EAP) to verify authorized

WLAN devices and/or users.

• Encryption for data that traverses a WLAN must use Advanced Encryption

Standard (AES) algorithms that are validated under the National Institute of

Standards and Technology (NIST) Cryptographic Module Validation Program 1 as

meeting requirements of FIPS 140-2, at a minimum Overall Level 1.

• WLAN products must be Wi-Fi Alliance, WPA2- certified.

• WLAN traffic must be logically segmented from wired network traffic by use of a

Virtual Local Area Network (VLAN), when possible.

2.2 Recommendations

The following recommendations will further enhance the security of a wireless

network. Where feasible, consider adopting these recommendations in addition to the

requirements listed above.

• Register the static IP addresses for wireless access points in the Domain Name

System (DNS), and use IHS naming conventions.

• Use MAC-based authentication by allowing only registered MAC addresses to

have access to the AP. It is possible to spoof MAC addresses, but MAC-based

authentication provides another layer to the security of an otherwise insecure data

transfer medium.

• Use multiple levels of authentication and security. Recommendations include

using Remote Authentication Dial-In User Service (RADIUS) authentication or

public key infrastructure (PKI).

• Ensure the wireless device satisfies any existing RFI standards.

• Use an encryption protocol between any two wireless devices. At a minimum, the

wireless access point must require WPA2 or connect through the IHS-managed

VPN concentrator. All encryption modules must be certified under FIPS 140-2.

1 The Cryptographic Module Validation Program is a joint effort between NIST and the Communications

Security Establishment Canada, that validates cryptographic modules to FIPS. Refer to

http://csrc.nist.gov/groups/STM/cmvp/index.html.

Indian Health Service  Version 2.2

Division of Information Security 6 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

• Avoid broadcasting to areas not intended for service. Locate APs centrally within

the intended area of service, and avoid placing APs near windows or against the

outside walls of buildings.

• Designated personnel should scan for unauthorized wireless APs (WAPs)

annually or when major changes are made. If a rogue WAP is found, the local

ISSO must be notified to investigate. (CAUTION: If a scan is conducted in a

hospital, the scanner could find devices that appear to be wireless access points

but are actually medical devices. Automatically disconnecting such a device is

dangerous and may lead to malfunction of medical devices.)

• Designated personnel should check annually to ensure that the organization’s

WLAN client devices and APs have configurations that are at all times compliant

with this SOP.

• Disable broadcasting the Extended Service Set Identification/Service Set

Identification (ESSID/SSID).

• Change all default ESSID/SSIDs from their default vendor settings to unique

names. The ESSID/SSID can be accessed from within wireless products’ web-

based or Windows-based configuration utilities. Common examples of predefined

ESSID/SSIDs are simple names like "wireless," "netgear," "linksys," or "default."

An ESSID/SSID can be changed at any time, as long as the change is also made

on all wireless clients. The following are best practices:

 Don't use information such as name, address, organization, or other types

of personal information as part of the ESSID/SSID.

 Don't use any of your Windows or Internet website passwords.

 Don't tempt would-be intruders by using tantalizing network names like

"HEALTHCARE-BOX" or "TOP-SECRET."

 Do pick an ESSID/SSID that contains both letters and numbers.

 Do choose a name as long or nearly as long as the maximum length

allowed.

 Do consider changing your ESSID/SSID every few months.

Indian Health Service  Version 2.2

Division of Information Security 7 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

3. Guidelines for Implementing Wireless Networks

at IHS

Follow these guidelines when implementing or using wireless networks.

• To enable wireless access for a user, the user’s supervisor must request the access

through the Information Technology Access Control (ITAC) system.

• Local IT staff will configure the client access securely, including encryption and

layered authentication.

• Local IT staff will maintain a database or equivalent system to track wireless

hardware allocation and user access.

• ISSOs and local IT staff should provide additional training on the risks associated

with wireless networks before they grant wireless privileges.

• IT staff will periodically audit connections and connection attempts and report

rogue hardware and security violations to the Area ISSO.

Indian Health Service  Version 2.2

Division of Information Security 8 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

4. Inappropriate Use

The following list describes inappropriate use of wireless networks.

• Unauthenticated or unencrypted wireless communications are not permitted on the

IHS network.

• The use of wireless keyboards is not allowed on IHS networks or equipment.

• Wireless communications must not be used to bypass IHS security controls such

as Websense and firewalls.

• Users are required to comply with the General User Security Handbook SOP06-

11a, which prohibits the insecure storage or transmission of sensitive data. Users

may not send, store, or process patient information without appropriate security

safeguards, as per HIPAA privacy and security requirements.

• Wireless devices are not to be used within an environment where they may

disrupt other systems (for example, telephone, healthcare equipment, and

radio/microwave transmitters).

• Per-session dynamic RC4 keys, including but not limited to dynamic wired

equivalent privacy (WEP), temporal key integrity protocol (TKIP), and WPA,

must not be used to secure WLAN communications.

• The use of static or pre-shared keys is not permitted.

• Wireless connections must not be used to administer the WLAN or LAN.

• Ad-hoc connections between endpoint wireless devices without a centralized

infrastructure are prohibited.

Indian Health Service  Version 2.2

Division of Information Security 9 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

5. Ch ecklist

Use the following checklist to ensure that wireless devices and networks are secure.

☐ Wireless device is installed in a properly secured, adequately monitored

location.

☐ Device IP address and supported IP address range is separate and unique from

the wired IP ranges; that is, there is no possibility of any wired device using

the WLAN IP addresses.

☐ Device configured with a static IP address.

☐ Device administration password changed from the default to a unique

password conforming to the IHS password policy.

☐ Device SSID changed from the default to a unique, unpublished SSID.

☐ Device configured not to broadcast the SSID.

☐ Device configured to automatically disable its wireless network capability

when physically plugged in through a port into the IHS enterprise wired

network.

☐ If SNMP is active, SNMP community strings changed from manufacturer

default to unique, unpublished strings.

☐ Device configured for WPA2 encryption. This is the minimum encryption

requirement.

☐ Device WLAN isolated by a firewall with the principle of least privilege

requirements (for example, specific IP addresses, protocols, ports).

☐ Inventory maintained and regularly updated with wireless devices,

configurations, and authorized users.

☐ Device signal area and frequency are on non-overlapping channels and do not

interfere with any other devices, wireless or otherwise (such as medical

devices).

☐ After they are connected to the device, users are authenticated to an IHS

central source, such as Active Directory.

☐ DNS entries conform to IHS naming conventions.

☐ Personnel are designated to periodically scan WLANs or monitor a wireless

intrusion detection system (IDS).

☐ Wireless users have been trained on the risks associated with using a wireless

device.

Indian Health Service  Version 2.2

Division of Information Security 10 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

6. Glossary

See the online glossary on the IHS Security website.

Indian Health Service  Version 2.2

Division of Information Security 11 February 2013

DIS-SOP-09-36 Wireless Network Security

Standards

7. Appendix: Wireless Scanning Tools

Name: VisiWave  Wireless Sniffer

Type:  Windows

Availability: Freeware, http://www.visiwave.com/

Description:  VisiWave Site Survey is a complete 802.11a/b/g/n site surveying

solution. It collects survey data using a standard wireless adapter while walking

around the survey area clicking on a floor plan image, using a GPS receiver to track

your location, or using a customized navigation device.

Name: AirSnort  Wireless Sniffer

Type:  UNIX

Availability: Freeware, http://airsnort.shmoo.com/

Description:  AirSnort is a freeware wireless LAN sniffer tool that recovers

encryption keys. AirSnort operates by passively monitoring transmissions, computing

the encryption key when enough packets have been gathered.

Name: Kismet  Wireless Sniffer

Type:  UNIX

Availability: Freeware, http://www.kismetwireless.net/

Description:  Kismet is an 802.11b wireless network sniffer. It is capable of

sniffing using almost any wireless card supported in Linux.

Name: Netstumbler  Wireless Sniffer

Type:  Windows

Availability: Freeware, http://www.netstumbler.com

Description:  Netstumbler is an 802.11b tool that listens for available networks

and records data about that access point. A version is available for the Pocket PC.

Name: WEPCrack  WEP encryption cracker

Type:  UNIX

Availability: Freeware, http://sourceforge.net/projects/wepcrack/

Description:  WEPCrack is a tool that cracks 802.11 WEP encryption keys using

the latest discovered weakness of RC4 key scheduling.

Name: WaveStumbler  Wireless Network Mapper

Type:  UNIX

Availability: Freeware, http://www.cqure.net/wp/tools/other/wavestumbler/

Description:  WaveStumbler is a freeware console-based 802.11 network

mapper for Linux. It reports the basic wireless network characteristics including

channel, WEP, ESSID, and MAC.

Indian Health Service  Version 2.2

Division of Information Security 12 February 2013

---

## Appendix: Compliance Framework Applicability

**Impact Level:** MODERATE  |  **Classification:** CUI

| Framework | Applicability |

|---|---|

| NIST SP 800-53 Rev 5 Moderate | Required |

| FedRAMP Moderate | Required |

| CMMC Level 2 | Required |

_Crosswalk coverage: N/A control mappings loaded._

_This document was generated by ICDEV™ Intelligent Documentation Regeneration (IDR)._

_Control implementation narratives: /compliance/crosswalk_

CUI